# Tutorial of NR_NTN-SIM: From constellation to 3D map (Paris → New York)

## Vue d'ensemble

We create a LEO constellation of 66 satellites, calculate its inter-satellite routes (ISL) between Paris and New York, launch NR-NTN-SIM on it, and then display the route on a 3D globe along with the simulation curves. Everything runs in Docker; allow approximately 3 hours the first time, including 2 hours of simulation (it advances by about 3 simulated seconds per minute).

In order we have :

- Image Docker nr-ntn-sim:full : ns-3.46 + 5G-LENA v4.2 + modules Hypatia, already compiled(Step&).

- Image du tutoriel nr-ntn-sim:tuto : The same, plus a few Python packages and the 3 scripts from the tutorial (step 2).

- Constellation : make_constellation.py calls satgenpy (Hypatia's generator) to write the orbits (TLE), ISL links, ground stations, then the fstate_*.txt routing tables over a 400s window which contains a route change (step 3).

- control of the generated topology (step4).

- Simulation NR-NTN-SIM : the 5G NR radio layer (5G-LENA) is coupled with the ISL routing read in the fstate (step 5).

- Map 3D : ntn_route_map.py replays the same fstate and draws the active route by phase on a Cesium globe (step 6).

- Curves : plot_results.py trace débit, SINR, délai et PDR de la simulation, avec les changements de route marqués (étape 7).

The result is: a reusable topology folder, simulation results (CSV, logs), an interactive HTML page and a PNG figure.

## Prerequisites and documents

You will need an Ubuntu PC (22.04 or later), 8 GB of RAM, 10 GB of free disk space, and an internet connection for steps 2 and 6.

Files to be retrieved from

| Fichier | Contenu | Taille |
| --- | --- | --- |
| nr-ntn-sim-full.tar.gz | The validated Docker image (ns-3.46, 5G-LENA v4.2, Hypatia) | ~2.8 GB once loaded |
| nrntn-tuto.zip | Dockerfile + make_constellation.py, ntn_route_map.py, plot_results.py | < 100 Ko |
| hypatia_data.tar.gz | Optional: paper topologies (for the variant) | ~30 Mo |

Working directory structure, to be created once. All commands in the tutorial are launched from ~/nr-ntn-sim:

mkdir -p ~/nr-ntn-sim/{hypatia_data,results,maps}
cd ~/nr-ntn-sim
unzip ~/Téléchargements/nrntn-tuto.zip -d .
ls                 # hypatia_data  maps  nrntn-tuto  results

| Dossier hôte | Seen in the container | contain |
| --- | --- | --- |
| hypatia_data/ | /data/hypatia | Topologies (constellation + fstate_*.txt) |
| results/ | /results | Simulation outputs |
| maps/ | /maps | HTML and CSV route maps |

The containers are launched with --rm : Anything not written in one of these three folders disappears upon exit.

## Step 1 : Install Docker et load the image

This only needs to be done once per machine. If docker run hello-world already working, proceed to "Load image".

Install Docker :

sudo apt update
sudo apt install -y docker.io
sudo usermod -aG docker $USER      # to launch Docker without sudo
# log out and then log back in 
docker run --rm hello-world        # must display "Hello from Docker!"

Load the image (2 to 5 min):

docker load -i ~/Téléchargements/nr-ntn-sim-full.tar.gz
docker images | grep nr-ntn-sim    # nr-ntn-sim   full   ...   2.8GB

## Step 2 : Build the tutorial image

The image full Satgenpy contains the package but not all of its Python packages (sgp4, geopy) nor matplotlib. Rather than reinstalling them for each container, a small derived image is built once, nr-ntn-sim:tuto, This adds the packages and copies the three scripts to /opt/nrntn-tools/. Duration: 1 to 3 minutes, internet connection required.

cd ~/nr-ntn-sim
docker build -t nr-ntn-sim:tuto nrntn-tuto

The construction must end with these two lines, otherwise it will stop with an error:

python deps OK
satgenpy OK

the Dockerfile (in nrntn-tuto/) do only this

FROM nr-ntn-sim:full
RUN pip3 install --no-cache-dir sgp4 geopy astropy ephem networkx numpy matplotlib
COPY make_constellation.py ntn_route_map.py plot_results.py /opt/nrntn-tools/

All subsequent steps use nr-ntn-sim:tuto. The simulator is identical to the full one. After modifying a script, rerun the same docker build command (a few seconds).

## Step 3: Create the constellation and routing tables

A single command creates a complete topology folder that can be used directly by the simulator; it takes approximately 1 minute.

cd ~/nr-ntn-sim
docker run --rm \
  -v "$PWD/hypatia_data":/data/hypatia \
  nr-ntn-sim:tuto \
  python3 /opt/nrntn-tools/make_constellation.py \
    --out /data/hypatia --name sh3_paris_newyork \
    --gs "Paris:48.8566:2.3522,New-York:40.7128:-74.0060"

### What the script does

1. Orbits. It writes tles.txt: 6 planes × 11 satellites, inclination 87.9°, 13.40 revolutions/day (≈ 1110 km altitude), epoch April 16, 2026. The plane distribution and phase shift are those of satgenpy.

2. Links and stations. satgenpy writes isls.txt (grid "+": 2 neighbors in the plane, 2 in adjacent planes, 132 links), ground_stations.txt (Paris = node 66, New York = node 67), and gsl_interfaces_info.txt.

3. Ground-to-satellite range. It is calculated for a minimum elevation of 10° (≈ 2970 km at this altitude): a satellite below the horizon is never used.

4. Route change search. It calculates the routing every second over a 2-hour orbit, identifies the first time the Paris → New York route changes, and positions the simulation window so that this change occurs at t ≈ 100 s.

5. Routing Tables. It calculates the window's fstates (400 s, 100 ms steps, shortest path on the ISLs), renames them so the window starts at fstate_0.txt, and writes them as actual files, without symbolic links.

6. Window Memory. It writes window.json: offset from the epoch, step size, altitude, and detected route changes. The map from step 6 automatically reads it.

### Expected output

Constellation : 6 orbits x 11 sats = 66 satellites, inc 87.9 deg, 13.4000 rev/day (~1109 km)
Ground links  : max 2968 km (min elevation 10.0 deg)
Route         : Paris (node 66) -> New-York (node 67)

[1/2] Scanning 7200 s at 1 s to find the first route change ...
      first route change at 356 s after the epoch -> window offset 256 s
[2/2] Generating 400 s of forwarding state at 100 ms ...

Done in 40 s  ->  /data/hypatia/sh3_paris_newyork
  4001 fstate files (fstate_0.txt = t 0 = 256 s after the epoch), window.json
  route at t=0  : [34, 23]  (1 ISL hops)
  route changes :
    t =    99.8 s   1 -> 1 ISL hops   [3, 58]
    t =   312.7 s   1 -> 1 ISL hops   [33, 22]

The figures come from a test with these parameters; we should find the same ones, since everything is deterministic. The resulting file is hypatia_data/sh3_paris_newyork/.

### Paramètres utiles

| Option | Default | Effect |
| --- | --- | --- |
| --orbits, --sats-per-orbit | 6, 11 | Size of the constellation |
| --inclination | 87.9 | Inclinaison of plans (°) |
| --mean-motion ou --altitude-km | 13.40 | Altitude (13.40 rev/day ≈ 1110 km) |
| --gs | Paris, New-York | Stations NAME:LAT:LON, the 1st = source, the 2nd = destination |
| --min-elev | 10 | Minimum elevation of ground links (°) |
| --duration, --step-ms | 400, 100 | Window and resolution of fstate (= --simTime) |
| --offset | auto | automatic, or a number of seconds after the time |
| --lead | 100 | With car: targeted time of the first change of route(s) |
| --force | non | Overwrites an existing folder |

The current simulator expects Paris at node 66 and New York at node 67. Therefore, keep 66 satellites, Paris first and New York second, until approach5 has been adapted.

## Step 4: Check the topology

Five checks, two minutes, directly on the host machine:

cd ~/nr-ntn-sim/hypatia_data/sh3_paris_newyork

ls | grep -v fstate # description.txt ground_stations*.txt gsl_interfaces_info.txt isls.txt tles.txt window.json

ls fstate_*.txt | wc -l #4001 (400s / 100ms + 1)

find . -type l | wc -l #0 (no symbolic link)

wc -l < ​​fstate_0.txt # 134 (complete table: 66 sat x 2 stations + 2 station->station lines)

cat window.json | head -20 # offset, altitude, route changes

Format of an fstate line:

current_node, destination, next_hop, output_interface, input_interface. Example: 66,67,34,0,4 means "from Paris (66), to New York (67), send to satellite 34". fstate_0.txt is complete; the following files only contain the lines that change, so most are empty (a few are complete: these are the beginnings of the blocks calculated in parallel).

To follow the route manually at t = 0:

grep '^66,67,' fstate_0.txt      # 66,67,34,...  -> first sat
grep '^34,67,' fstate_0.txt      # 34,67,23,...  -> next satellite
grep '^23,67,' fstate_0.txt      # 23,67,67,...  -> to New York

Step 5: Start the simulation

First, a 30-second simulation is run to validate the chain (a few minutes), then the full 400 seconds are run in the background (approximately 2 hours; measured: 30 seconds simulated ≈ 10 minutes).

To monitor progress: `tail -1 results/SH3/timeseries_alt1110_pwr55.csv | cut -d, -f1` displays the simulated time reached.

Do not rerun the command while `docker ps -a --filter name=sh3` shows `Up`: the name `sh3` is already taken, and a `docker rm -f sh3` would terminate the current run.

### 5a. Short Test(30 s)

cd ~/nr-ntn-sim
docker run --rm -it \
  -v "$PWD/hypatia_data":/data/hypatia \
  -v "$PWD/results":/results \
  nr-ntn-sim:tuto

We are now inside the container, in /opt/ns-3.46 :

mkdir -p /results/SH3_test
./ns3 run "ns3_nrntnsim_timeseries_optionD_approach5 --satAlt=1110 --txPower=55 --lambdaDl=157000 --lambdaUl=2000 --simTime=30 --hypatiaDir=/data/hypatia/sh3_paris_newyork/ --outputDir=/results/SH3_test/"
exit

To check:

• [HYPATIA] lines appear, with the number of hops and the delay;

• The destination is node 67 (New York). A different destination means the image doesn't have the DST_GS patch: notify Joe;

• `ls results/SH3_test/` on the host machine shows files, including a `timeseries_*.csv`.

### 5b. Complete simulation (400 s)

cd ~/nr-ntn-sim
docker run -d --name sh3 \
  -v "$PWD/hypatia_data":/data/hypatia \
  -v "$PWD/results":/results \
  nr-ntn-sim:tuto \
  bash -c 'mkdir -p /results/SH3 && ./ns3 run "ns3_nrntnsim_timeseries_optionD_approach5 --satAlt=1110 --txPower=55 --lambdaDl=157000 --lambdaUl=2000 --simTime=400 --hypatiaDir=/data/hypatia/sh3_paris_newyork/ --outputDir=/results/SH3/"'

docker logs -f sh3        # follow the progress ; Ctrl+C to stop the display not the simulation

Finally, keep the log and delete the container:

docker logs sh3 > results/SH3/run.log 2>&1
grep HYPATIA results/SH3/run.log > results/SH3/hypatia_events.log
docker rm sh3
ls results/SH3/

The paper used `--satAlt=1200` with a constellation at approximately 1110 km. Here, the two are aligned; to compare with the paper's figures, run the same command again with `--satAlt=1200`.

The files created in `results/` belong to root. To modify them: `sudo chown -R $USER ~/nr-ntn-sim/results`.

## Step 6: Visualize the route on the 3D globe

ntn_route_map.py It reads the same fstates as the simulator, identifies each route change, recalculates the propagation delay of each route from the TLEs (same method as Hypatia), and produces an HTML page. The window offset is read from window.json; there is nothing to configure. Duration: a few seconds.

cd ~/nr-ntn-sim
docker run --rm \
  -v "$PWD/hypatia_data":/data/hypatia:ro \
  -v "$PWD/maps":/maps \
  nr-ntn-sim:tuto \
  python3 /opt/nrntn-tools/ntn_route_map.py \
    --fstate-dir /data/hypatia/sh3_paris_newyork \
    --title "SH3 Paris → New York" \
    --out /maps/sh3_paris_newyork.html

Exected output  (same try as step3 3) :

Offset        : 256.0 s (from window.json)
Route         : Paris (node 66) -> New-York (node 67)
Mean altitude : 1121 km (from the TLEs, at t=0)
fstates       : 4001 files, step 0.100 s, window 0.0-400.0 s, offset 256.0 s

  #   t_start    t_end   ISL hops  delay[ms]  elev src/dst[deg]  satellites
  1       0.0     99.8        1      29.72        15.8 / 41.0     [34, 23]
  2      99.8    312.7        1      31.08        11.6 / 33.9     [3, 58]
  3     312.7    400.0        1      34.69        18.8 / 29.5     [33, 22]

Each line represents a phase: start, end, number of ISL hops, forward propagation delay, elevation angle of the two ground links, and satellites traversed. The times must be those announced by make_constellation.py.

Open the page (internet access required for the Cesium library and imaging):

cd ~/nr-ntn-sim/maps
python3 -m http.server 8080 
# in Chrome : http://localhost:8080/sh3_paris_newyork.html
# The server is running in the background; to stop it: pkill -f "http.server"

The script also writes maps/sh3_paris_newyork_timeline.csv (route and delay at each step) and maps/sh3_paris_newyork_phases.csv (one line per phase), useful for step 7.

Options: --times 0,100,300 shows selected times instead of detected changes; --src and --dst change the direction.

## Step 7 : Plot the simulation results

plot_results.py reads the timeseries_*.csv file written by the simulation (one line every 0.1 s) and produces a four-panel figure: DL/UL throughput, SINR DL, delays, PDR. Route changes from window.json are plotted in red lines, and the propagation delay calculated in step 6 is superimposed in black dashed lines.

For simulation 5b:

cd ~/nr-ntn-sim
docker run --rm \
  -v "$PWD/hypatia_data":/data/hypatia:ro \
  -v "$PWD/results":/results \
  -v "$PWD/maps":/maps:ro \
  nr-ntn-sim:tuto \
  bash -c 'CSV=$(ls /results/SH3/timeseries_*.csv | head -1) && python3 /opt/nrntn-tools/plot_results.py \
    --csv "$CSV" --warmup 10 \
    --window /data/hypatia/sh3_paris_newyork/window.json \
    --map-timeline /maps/sh3_paris_newyork_timeline.csv \
    --title "SH3 Paris → New York" --out /results/SH3/sh3_timeseries.png'

For simulation 5a:

cd ~/nr-ntn-sim
docker run --rm \
  -v "$PWD/hypatia_data":/data/hypatia:ro \
  -v "$PWD/results":/results \
  -v "$PWD/maps":/maps:ro \
  nr-ntn-sim:tuto \
  bash -c 'python3 /opt/nrntn-tools/plot_results.py \
    --csv /results/SH3_test/timeseries_alt1110_pwr55.csv --warmup 10 \
    --window /data/hypatia/sh3_paris_newyork/window.json \
    --map-timeline /maps/ns3_paris_newyork_timeline.csv \
    --title "SH3 Paris → New York" --out /results/SH3_test/sh3_timeseries.png'